SECTION 10 71 13 - WINDOW SHADES

Polar Shades Commercial Sun Control - ZipRite

Polar Shades; window shade products.

This section is based on the products of Polar Shades, which is located at:

Polar Shades

5520 Stephanie St.

Las Vegas, NV 89122

GENERAL

SECTION INCLUDES

Motorized Exterior Roller Shade ZipRite System: Electrically powered, retractable fabric roller shade assembly complete with housing, side guides, drive motor, control components, and all necessary mounting accessories.

  - RELATED SECTIONS

Please add or remove sections as your project requires.

Section 061000 “Rough Carpentry:”

Section 079200 “Joint Sealants:”

Section 084413 “Glazed Aluminum Curtain Walls:”

Section 084423 “Structural-Sealant-Glazed Curtain Walls:”

Section 084113 “Aluminum-Framed Entrances and Storefronts:”

Section 084126 “All-Glass Entrances and Storefronts:”

Section 085000 “Windows:”

Section 085113 “Aluminum Windows:”

Section 085123 “Steel Windows:”

Section 085200 “Wood Windows:”

Section 085313 “Vinyl Windows:”

Section 085413 “Fiberglass Windows:”

Division 26 - Electrical: Electrical power, conduit, and wiring for motorized roller shades.

REFERENCES

Please remove any references that are not required.

NFPA 70 - National Electrical Code.

NFPA 701 - Fire Tests for Flame-Resistant Textiles and Films.

CA TITLE 19 – CA Standard for fire tests of flame-resistant textiles

GREENGUARD Gold

US Green Building Council (USGBC).

SUBMITTALS

Submit under provisions of Section 01 33 26 “Submittal Procedures:”

Product Data: Manufacturer's data sheets on each product specified, including:

Provide manufacturer instructions for installation and ongoing maintenance.

Include guidelines for proper storage, handling, and recommended practices.

Submit drawings showing mounting configurations and approved installation techniques.

Provide standard wiring schematics illustrating connection of motor controls with building automation systems and/or third-party control platforms.

Please include if the following is applicable to your project, delete if not required

Shop Drawings: Floor plans, elevations, cross-sections, product-specific details, installation requirements, operational clearance diagrams, electrical wiring layouts, and coordination with adjacent construction.

Please remove the following if LEED is not applicable.

LEED Submittals: Submit documentation demonstrating compliance with the requirements for obtaining the specified Credit.

Window Treatment Schedule: For all roller shades, use the room designations as shown on the drawings, along with verified window dimensions, quantities, shade types, control systems, fabric selection, and color. Include details for opening sizes and provide a key to typical mounting configurations.

Selection Samples: For each finished product specified, two complete sets of shade cloth options and aluminum finish color samples representing manufacturer's full range of available colors and patterns.

Verification Samples: For each specified finished product, submit two complete sets of unassembled shade components to demonstrate compliance with the specified requirements. Include shade fabric and aluminum finish samples as selected, accurately representing the final product, including color and pattern. Clearly mark the face of the materials to indicate interior surfaces.

Maintenance Data: Provide maintenance procedures for motorized exterior ZipRite shades, including recommended cleaning materials and techniques. Include operating instructions for hardware and control systems, along with necessary precautions.

Manufacturer's Certificates: Provide certification that products comply with or exceed the specified requirements.

QUALITY ASSURANCE

Manufacturer Qualifications: Procure motorized exterior window ZipRite shades from a single manufacturer with proven experience in producing products similar to those specified in this section.

NFPA Flame-Test: Passes NFPA 701. Materials used in testing must be identical to those proposed for the final installation.

Mock-Up: Submit a mock-up of each type of motorized exterior ZipRite roller shade assembly specified, for evaluation of mounting configurations, appearance, and included accessories.

Position the mock-up in the window(s) as designated by the Architect.

Do not continue with the remaining work until the mock-up has been approved by the Architect.

DELIVERY, STORAGE, AND HANDLING

Do not deliver the exterior motorized ZipRite roller shades until the building is fully enclosed and construction within the areas where the shades will be installed is nearly complete.

Deliver products in the manufacturer’s original, sealed, and undamaged packaging, ensuring labels remain intact.

Label all containers and shades in accordance with the Window Shade Schedule.

Store products in the manufacturer’s original, unopened packaging until ready for installation.

PROJECT CONDITIONS

Install exterior motorized ZipRite roller shades only after the window finishing work has been completed.

WARRANTY

Hardware and Shade Fabric: Covered under the Polar Shades Commercial Sun Control Standard Lifetime Limited Warranty.

Motors and Controls: Polar Shades Commercial Sun Control standard five-year limited warranty.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Polar Shades Commercial Sun Control, 5520 Stephanie St., Las Vegas, NV 89122, Christopher Hagen, 972.467.2222, chrish@polarshade.com

Requests for substitutions will be considered in accordance with provisions of Section 01 33 26 "Substitution Procedures.”

EXTERIOR MOTORIZED ZIPRITE SHADE SYSTEM

Roller Tube: Fabricated from galvanized steel, 3.07 inches (78 mm) in diameter (at manufacturer’s discretion) to accommodate shade type and size. Fabric connects to the roller tube groove assembly with a fabric spline.

Headbox: Consists of cassette-style extruded aluminum sections with endcaps.

Select one or more of the following headbox sizes and shapes. If selecting more than one, be sure to indicate clearly on the drawings or describe clearly in the specifications where headboxes of each size and shape are located.

Elite ZipRite Open Roll: 4.75” height x 4.5” depth; for shades up to 14’0” wide x 10’0” tall

Elite ZipRite with Pocket: 5.125” height x 5.5” depth, for shades up to 14’0” wide x 10’0” tall

Elite ZipRite with 4.5” Cassette: 4.5” height x 4.625” depth, for shades up to 14’0” wide x 10’0” tall

Patio Roll ZipRite Open Roll: 6.125” height x 6.75” depth, for shades up to 20’0” wide x 14’0” tall

Titan ZipRite Cassette: 5.5” height x 5.625” depth, for shades up to 20’0” wide x 14’0” tall

Patriot ZipRite Open Roll: 6.125” height x 6.75” depth, for shades up to 20’0” wide x 14’0” tall

Mega ZipRite with Cassette: 6.3125” height x 5.9375” depth, for shades up to 24’0” wide x 14’0” tall

Mega ZipRite with Patriot Hood: 6.125” height x 6.75” depth, for shades up to 24’0” wide x 14’0” tall

Mega ZipRite with Pocket: 9” height x 7.25” depth, for shades up to 24’0” wide x 14’0” tall

Endcaps: Aluminum end brackets with integrated lugs for attaching the headbox to side channels. Provide brackets sized to match the dimensions of the headbox.

ZipRite Side Channels: 2.75” x 1.5

Hembar must be captured inside the side channel, an uncaptured or “free” hembar alternative is not acceptable.

Finish: White, Bronze, Sand, Camel, Tan, Black; custom powder coated finishes are available.

U-Channel 2” x 1.75” required for inside mount, when mounting between an opening rather than face-mounting to a wall or wall-mounting to a window frame.

Hem bar: Hembar must be captured inside the zipper track as the fabric rolls to an open or closed position. Uncaptured hembars are not acceptable.

Zipper: Double-flap welded Keder, bonded on both sides.

Idle End Assembly: Spring-loaded steel bracket offering a bottom loading idle-end pin, insuring positive-locked attachment while making it easier to install or uninstall the shading system.

Shade Motor.

Standard Motor: 110VAC tubular motor with 4-wire pigtail. All controls are wired to the motor. This system can be tied into building management systems and third party control and integration systems.

Standard Motor: 110 VAC, 60 HZ, reversible, lifetime lubricated, and equipped with internal thermal overload protector, electric brake, and pre-set accessible limit switches. Tubular motor concealed inside each shade roller tube.

Radio Technology Motor: Optional Motor with built-in radio receiver. 3-wire pigtail allows for parallel wiring. This system can be integrated into building management systems and with third party control systems. All controls are “wireless” RF communication.

          - Radio Technology Motor- 110 VAC motor with built-in radio receiver. Tubular motor concealed inside each roller tube assembly.

        - RTS Controls

Please select controls from the following list and remove any controls that are not needed.

          - Decora RTS 1-5 Channel Wall Switch Kit

          - Situo 1 Single Channel Handheld Remote (indoor only)

          - Telis 1 Single Channel Handheld Remote (indoor only)

          - Situo 5 Five Channel Handheld Remote (indoor only)

          - Telis 4 Five Channel Handheld Remote (indoor only)

          - Telis 16 Sixteen Channel Remote (indoor only)

          - Telis 1 Patio Remote

          - Telis 4 Patio Remote

          - Eolis RTS Wind Sensor

          - Soliris RTS Wind/Sun Sensor

          - Situo 5 Soliris Remote

          - TaHoma (indoor only)

        - Standard Motor Controls:

IGC4n1 by Somfy, each unit controlling up to four 110VAC motors. Multiple IGC4n1s can be daisy chained together to create a larger network of interconnected motorized shades, allowing a single controller to operate multiple motors on the project.

FABRIC

Light-Filtering and Room Darkening Fabrics: Choose from a selection of fabrics from Phifer, Mermet, Twitchell, as well as our PolarTech Fabrics and PolarTech Performance Fabrics.

Custom Printed Graphics: Available as submitted to manufacturer in accordance with manufacturer’s guidelines.

EXECUTION

EXAMINATION

Do not commence installation until all substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify the Architect of any unsatisfactory conditions before proceeding with installation.

PREPARATION

Coordinate requirements for blocking and structural supports to ensure adequate means for installation of window shades.

Coordinate the requirements for power supply conduit and wiring necessary for the exterior motorized window ZipRite shades and their control systems.

INSTALLATION

Install according to the manufacturer’s guidelines.

Install exterior motorized window ZipRite shades level, plumb, square, and aligned. Ensure adequate clearances for window operation hardware.

Position shades level, plumb, and at the correct height relative to adjacent construction. Secure in place using fasteners recommended by the manufacturer.

Include the following paragraph if systems are intended for high levels of light exclusion. Delete if not applicable.

Install headbox, side channels, and sill channel with the sealant specified in Section 079200 “Joint Sealants” to prevent light leakage around the perimeter of the shade system.

TESTING AND DEMONSTRATION

Test exterior motorized window ZipRite shades to ensure the operating mechanism, fabric retainer, and all other components are functioning correctly. Address any deficiencies.

Demonstrate the operation of the shades to the Owner’s designated representatives.

PROTECTION

Protect installed products until the project is complete.

Touch up, repair, or replace any damaged products prior to Substantial Completion.

SCHEDULES

Retain the paragraph below as applicable to the project. Identify products by name on the Drawings, or alternatively, use this paragraph to specify the location of each material type to be used. Delete if not required.

Refer to Drawings for ZipRite exterior motorized Roller Shades types and locations.

END OF SECTION