SECTION 12 24 00 - WINDOW SHADES

Polar Shades Commercial Sun Control – Interior Manual Single Roller Shade

Polar Shades; window shade products.

This section is based on the products of Polar Shades, which is located at:

Polar Shades

5520 Stephanie St.

Las Vegas, NV 89122

GENERAL

SECTION INCLUDES

Interior Manual Single Roller Shade System: Manual fabric roller shades for interior applications, including all supporting hardware and operating mechanisms

  - RELATED SECTIONS

Please add or remove sections as your project requires.

Section 061000 Rough Carpentry

Section 079200 Joint Sealants

Section 092123 – Gypsum Board Assemblies: Suspended ceilings constructed of gypsum board incorporating recessed pockets for window shade systems

Section 09510 [09 51 23] - Acoustical Tile Ceilings

REFERENCES

Please remove any references that are not required.

NFPA 70 - National Electrical Code.

NFPA 701 - Fire Tests for Flame-Resistant Textiles and Films.

CA TITLE 19 – CA Standard for fire tests of flame-resistant textiles

GREENGUARD Gold

US Green Building Council (USGBC).

ANSI/WCMA A100.1-2022: Manual window shades utilizing closed-loop bead chains shall comply with all current Consumer Product Safety Commission requirements.

SUBMITTALS

Submit under provisions of Section 01 33 26 “Submittal Procedures:”

Product Data: Manufacturer's data sheets on each product specified, including:

Provide manufacturer instructions for installation and ongoing maintenance.

Include guidelines for proper storage, handling, and recommended practices.

Submit drawings showing mounting configurations and approved installation techniques.

Please include if the following is applicable to your project, delete if not required

Shop Drawings: Floor plans, elevations, cross-sections, product-specific details, installation requirements, operational clearance diagrams, electrical wiring layouts, and coordination with adjacent construction.

Please remove the following if LEED is not applicable.

LEED Submittals: Submit documentation demonstrating compliance with the requirements for obtaining the specified Credit.

Window Treatment Schedule: For all roller shades, use the room designations as shown on the drawings, along with verified window dimensions, quantities, shade types, control systems, fabric selection, and color. Include details regarding opening sizes and provide a key to typical mounting configurations.

Selection Samples: For each finished product specified, two complete sets of shade cloth options and aluminum finish color samples representing manufacturer's full range of available colors and patterns.

Verification Samples: For each specified finished product, submit two complete sets of unassembled shade components to demonstrate compliance with the specified requirements. Include shade fabric and aluminum finish samples as selected, accurately representing the final product, including color and pattern. Clearly mark the face of the materials to indicate interior surfaces.

Maintenance Data: Provide maintenance procedures for manual interior roller shades, including recommended cleaning materials and techniques. Include operating instructions for hardware and control systems, along with necessary precautions.

Manufacturer's Certificates: Provide certification that products comply with or exceed the specified requirements.

QUALITY ASSURANCE

Manufacturer Qualifications: Procure manual interior roller shades from a single manufacturer with proven experience in producing products similar to those specified in this section.

NFPA Flame-Test: Passes NFPA 701. Materials used in testing must be identical to those proposed for the final installation.

Mock-Up: Submit a mock-up of each type of manual interior roller shade assembly specified, for evaluation of mounting configurations, appearance, and included accessories.

Position the mock-up in the window(s) as designated by the Architect.

Do not continue with the remaining work until the mock-up has been approved by the Architect.

DELIVERY, STORAGE, AND HANDLING

Do not deliver the interior manual roller shades until the building is fully enclosed and construction within the areas where the shades will be installed is nearly complete.

Deliver products in the manufacturer’s original, sealed, and undamaged packaging, ensuring labels remain intact.

Label all containers and shades in accordance with the Window Shade Schedule.

Store products in the manufacturer’s original, unopened packaging until ready for installation.

PROJECT CONDITIONS

Install interior manual roller shades only after the window finishing work has been completed.

WARRANTY

Hardware and Shade Fabric: Covered under the Polar Shades Commercial Sun Control Standard Lifetime Limited Warranty.

PRODUCTS

MANUFACTURERS

Acceptable Manufacturer: Polar Shades Commercial Sun Control, 5520 Stephanie St., Las Vegas, NV 89122, Christopher Hagen, 972.467.2222, chrish@polarshade.com

Requests for substitutions will be considered in accordance with provisions of Section 013326 "Substitution Procedures.”

  - INTERIOR MANUAL ROLLER SHADE SYSTEM

        - Manually operated interior roller shade, clutch-operated Polar Shade Interior Manual Roller Shade as manufactured by Polar Shades in Las Vegas, NV.

Operation: Shades shall be operated by a bead chain and clutch mechanism that holds position when the chain is released. System shall be maintenance-free, requiring no adjustment or lubrication. Incorporate limit stops to restrict over-travel in both directions.

Minimum breaking strength of the chain shall be 100 pounds of pull force; anything less will not be accepted.

        - Clutch Mechanism: Corrosion resistant glass reinforced polyester thermopolymer for wear resistance and smooth operation.  Internal braking mechanism shall be a bi-directional wrap spring.  Provide a heavy-duty spring that creates a positive mechanical relationship between the roller shade tube unit and the universal installation brackets to ensure stationary positioning in the static state.  When activated the wrap spring shall release and permit the clutch to turn while reducing friction on the clutch.

        - Roller Tube: Fabricated from T6 extruded aluminum (T5 aluminum shall not be accepted), 1.5 inches (37 mm) in diameter (at manufacturer’s discretion) to accommodate shade type and size. Fabric connects to the roller tube groove assembly with a fabric spline. Larger / wider shades may require a 50mm (2.0”) or 63mm (2.5”) tube.

        - Spring-Loaded Idle End Plug: The idle end of clutch shall be designed to provide secure anchorage into end bracket

        - Hembar: Three-Sided Welded Pocket (Bottom Weight): Aluminum extrusion one inch in height held inside a fabric bottom-pocket.  The bottom-pocket shall be created by folding a 1 1/4-inch section of fabric behind the bottom of the shade and RF or impulse welding the fabric to itself.  Upon request, after the aluminum weight is inserted into the bottom-pocket the edges shall be sealed by again RF or impulse welding the fabric edges to itself.

Top Treatment / Valance

Select one or more of the following top treatment sizes and shapes. If selecting more than one, be sure to indicate clearly on the drawings or describe clearly in the specifications where headboxes of each size and shape are located. Some sizes are dictated by the size of the shade.

Extruded Aluminum Pocket – 4.5” x 4”, 5.5” x 5”, 7” x 9”, and custom sizes and colors available upon request. Each pocket size includes an option to add a ¾” horizontal lip at the bottom to provide integration with a drop-down ceiling tile system. Standard colors available are White and Anodized Aluminum.

Bottom Enclosure Only – Available in White or Anodized Aluminum colors as a standard, custom colors available upon request. Available sizes are 2.125, 2.625, 3.125, and 4.125 inches.

Extruded Aluminum Fascia – Available in 3” or 4” sizes. Some shades, if large or wide enough, will require 4” only. Standard colors available are White, Vanilla, Black, Bronze, and Anodized Aluminum.

Exposed / Open Roll – A cost effective solution that still presents a handsome profile utilizing our color-coordinated hardware and clean aesthetics on our components. Clutch and hardware colors are available in White or Black, with many more colors available in our premium selection.

Endcaps: Color-coordinated and available on all pocket, fascia, and open roll bracket solutions.

FABRIC

Light-Filtering and Room Darkening Fabrics: Choose from a selection of fabrics from Yuma, Phifer, Mermet, DongWon, Twitchell, as well as our PolarTech Fabrics and PolarTech Performance Fabrics.

Custom Printed Graphics: Available as submitted to manufacturer in accordance with manufacturer’s guidelines.

PVC-free and reclaimable options are available

EXECUTION

EXAMINATION

Do not commence installation until all substrates have been properly prepared.

If substrate preparation is the responsibility of another installer, notify the Architect of any unsatisfactory conditions before proceeding with installation.

PREPARATION

Coordinate requirements for blocking and structural support to ensure adequate means for installation of window shades.

INSTALLATION

Install according to the manufacturer’s guidelines.

Install interior manual roller shades level, plumb, square, and aligned. Ensure adequate clearances for window operation hardware.

Position shades level, plumb, and at the correct height relative to adjacent construction. Secure in place using fasteners recommended by the manufacturer.

Include the following paragraph if systems are intended for high levels of light exclusion. Delete if not applicable.

Install headbox, side channels, and sill channel with the sealant specified in Section 079200 “Joint Sealants” to prevent light leakage around the perimeter of the shade system.

TESTING AND DEMONSTRATION

Test interior manual roller shades to ensure the operating mechanism, fabric retainer, and all other components are functioning correctly. Address any deficiencies.

Demonstrate the operation of the shades to the Owner’s designated representatives.

PROTECTION

Protect installed products until the project is complete.

Touch up, repair, or replace any damaged products prior to Substantial Completion.

SCHEDULES

Retain the paragraph below as applicable to the project. Identify products by name on the Drawings, or alternatively, use this paragraph to specify the location of each material type to be used. Delete if not required.

Refer to Drawings for interior manual Roller Shade types and locations.

END OF SECTION